# Annex V: Itemised Unit Investment and Operating Costs

| Item | Capacity/ quantity | Unitary value (€) | Comments |
| --- | --- | --- | --- |
| Investment costs |  |  |  |
| Landfill Construction |  |  |  |
| Construction of first cell of the landfill* (composed of 1 cells) with a capacity of 184.500 m³ for the first cell | 184500m3 | 26 | Incl. all general landfill-infrastructure, gas- and leachate collection for cell 1, treatment fur full capacity, without access road. |
| Landfill/dumpsite Closure |  |  |  |
| Temporary / Final Closure of compliant landfill*, first cell, with an area of x ha | n/a | n/a |  |
| Final Closure of 2 (non-compliant) urban landfills in the whole county | 2 landfills | ~1,60M EUR/landfill | €2370k for the environmental cleansing of Dragasani landfill; €825k for the environmental cleansing of Calimanesti landfill; |
| Sorting Plant |  |  |  |
| Construction of 3 new sorting plants* for sorting of separately collected PW | up to 41 kt/a | 240,13 EUR/ton | €2,22m for the construction of Brezoi sorting plant; €3,98m for the construction of Raureni sorting plant; €3,65m for the construction of Roesti sorting plant; |
| MBT plant |  |  |  |
| MBT, with (pre-)sorting and biology | up to 34 kt/a | 152,86 EUR/ton | The content of the wet bin and the market waste will be delivered in the new central simple MBT/composting plant for the production of stabilized product (Compost like Output). The plan will result in the production of approximately 30,800 tn/year of compost like output (CLO), which will be utilized as cover material in the landfill, or for the recultivation of existing non compliant landfills, old landfills and other contaminated areas, or in forests (unit cost based on capacity utilisation for 29 years). |
| Collection |  |  |  |
| Bins, Containers: Various capacities | bins | 122 EUR/pce | Bins/ containers sizes, and subsequently costs, vary dependant on location (urban / rural). |
| Bins, Containers: Home Composting Bins | 22200 bins | 50 EUR/pce | Bins/ containers sizes, and subsequently costs, vary dependant on location (urban / rural). |
| O&M Costs |  |  |  |
| Landfill | 66 kt/a | 27,25 EUR/ton | Following the construction of the MBT facility and sorting plant an average of 66k tonnes will go to landfill. |
| Post Closure monitoring and control | 2 landfills | 5,000 EUR/yr/landfill | This provision is assumed to commence in 2014. |
| Sorting Plant | up to 41 kt/a | 22,2 EUR/ton | Based on total capacity of 41.000 tonnes per annum. |
| Transfer stations (total capacity of 26.000 tons) and haul to final destinations | up to 26 kt/a | 6,16 EUR/ton | Based on total capacity of 26k tonnes per annum. |
| Collection costs (residue waste and/or PW) | 96 kt/a | 25,82 EUR/ton | Collection costs calculated per tonne, with no differention assumed between residual and PW. |
| Savings & External Revenues |  |  |  |
| Paper & Cardboard | 10864 t/a | 40 EUR/ton |  |
| Plastics | 6466 t/a | 50 EUR/ton |  |
| Metal | 3103 t/a | 160 EUR/ton |  |
| Glass | 3939 t/a | 2 EUR/ton |  |
| Compost | 31400 t/a | 3,5 EUR/ton | Assumes 10% is sold . |
| Energy | n/a | n/a |  |
| Residential User Fees | 96406 t/a | 78 EUR/ton | €51 per tonne in 2011 increasing to the DPC by 2038 |
| Economic User Fees | 35324 t/a | 94 EUR/ton | €58 per tonne in 2011 increasing to the DPC by 2013. |